VxWorks System Boot

Copyright 1984-1993 Wind River Systems, Inc.

| CPU: Motorola MVME162 Version: 5.1 BSP version: 1.0 Creation date: Thu Mar 31 14:18:46 PST 1994 |  |
| --- | --- |
| Press any key to stop auto-boot... 0 auto-booting... |  |
| boot device : ei processor number : 0 file name : /tftpboot/vxBoot/vxWorks inet on ethernet (e) : 10.0.0.2:ffffff00 host inet (h) : 10.0.0.1 user (u) : vnmr1 flags (f) : 0xca startup script (s) : /vnmr/acq/vwScript Attaching network interface ei0... done. Getting boot parameters via network interface ei0. Bootp Server:10.0.0.1 Boot file: /tftpboot/vxBoot/vxWorks Attaching network interface lo0... done. Loading... Transfer Timed Out. 10.0.0.1:/tftpboot/vxBoot/vxWorks: tftp transfer failed: error 0x4b0007 Error loading file: errno = 0x44. Can't load boot file!! | Check: /etc/inetd.conf - Check for any characters in front of ‘tftp---‘ - Check if ‘bootpd’ is running. - If any fixes are made, reboot the Sun. - Start ‘bootpd’ - Start /vnmr/acqbin/bootpd –s - Check for /etc/bootptab Sun is waiting for a tftp request from the Acquisition CPU. |
| [VxWorks Boot]: |  |